SeaRey Adventures

Progressive Aerodyne SeaRey Flight Training

Normal Water Takeoff Procedure

GIFFTS

Gear- Selected/Indicated/Verified up for water

Instruments-“in the green”

Flaps-30 degrees

Fuel Pumps-Both On

Trim-Full Aft

Situational Awareness-Winds/Boats/Waves/Terrain

-Begin Takeoff with stick full aft.  As aircraft comes on to step release back pressure or light forward stick.

-Hold takeoff pitch attitude until aircraft lifts off.

-Flaps 20 and climb at Vx-58 mph (if needed to clear obstacle).

-Confirm gear up.

-Over any obstacles, flaps 10 and climb at Vy-63 mph.

-Reduce power below yellow arc on tachometer.

-Level off at 500’ agl, flaps up and reduce power to maintain 80 mph

(approx. 4400 rpm) if staying in the pattern.

Normal Water Landing Procedure

GIFFTS (on downwind)

Gear- Selected/Indicated/Verified up for water

Instruments-“in the green”

Flaps-Up

Fuel Pumps-Both On

Trim-As required

Situational Awareness-Winds/Boats/Waves/Terrain/ Obstacles/Powerlines

-Abeam landing point, 3600-3800 RPM, Flaps 10, 75 mph, “Gear up for water”.

-On Base, Flaps 20, 70 mph, “Gear up for water”.

-On Final, Flaps 30, 65 mph, “Gear up for water”.

-Hold pitch attitude, reduce power slightly, touchdown at 48-52 mph.

-On touchdown, slight forward stick and reduce power to idle.

-Forward stick as aircraft slows, wings level.

-As aircraft comes off step full aft trim.