SeaRey Adventures

ASES Ground School

-Check License/Medical/Logbook/Drivers License (fill out back of 8710-11 D.L. portion)

-What LSA-ASES allow you to fly

Searey/Icon/SeaMax/Cub/Champ etc.

-Buoys 		Red Right Returning to land or upstream

Can Buoys on left/Nun buoys on right-returning from sea

Can buoys solid black or green returning from sea

-Seaplane base chart markings

-Parts of a seaplane/floatplane

Keel

Sponsons

Bow

Stern

Step

Chines

Tip Floats

-Hydrodynamic lift vs. aerodynamic lift

-Purpose of step-reduces drag. w/out step will never leave water

-Step taxiing with wind. More power (and groundspeed) required for downwind than upwind due to loss of aerodynamic lift.

-Windspeed vs. waves  	1-3 Ripples

4-7 wavelets

8-12 wavelets w/breaking crests (foam or small whitecaps)

13-18 waves with frequent whitecaps

-Wind direction determination	Calm area/waves perpendicular/wind streaks greater than 8 mph/smoke/flags/weathervaning

-Taxi Types   Idle or Displacement/Plow(to turn out of wind/anything above idle)/Step

-Danger Turn-Downwind to upwind plow taxi (bury float)

-Sailing	Why?  Allows you to back into parking area.  Mainly floatplane maneuver.  Always done into the wind.

-Step Taxiing-Pitch range is everything  Small pitch range

-Getting on step

Normally accomplished with full aft stick

Forward stick needed

Aft C.G.

Heavy Weight

Tailwind

Hi Density Altitude

-Porpoise-SeaRey normally due to nose low/bow wave.  Immediately increase pitch angle to normal range

-Skipping-Touchdown too fast and therefore too nose low.

-T.O./Landing distance.  Shorter landing than takeoff normally-beware

-Normally land into wind…no wind and river land with current.  Wind overrides current.

-ROW 91.115  A/C give way as much as possible.. Other ROW rules similar

-Ramping procedures

Bow wave cushions impact with ramp

Flaps up tailwind/Flaps down headwind

Single gear as necessary

X-Wind technique

Have an out!!

-Hi-thrust line characteristics

Go-around

Engine Failure

Touch and Go

Full aft trim all takeoffs.

-C.G. Characteristics

Getting on step

Experimental vs S-LSA

-Ramping/Docking (HHH-Hatch, headsets, harness)

-W.O.S.T checks (required because there are way more ways to screw up than airport ops)

Wind

Obstructions (above and below)

Size

Taxi Plan

-Airplane

-Normal T.O.’s Discussion

-Wings Level

-Quicker on Power

-Forward stick as required

-Normal Pattern discussion

-GIFFTS

-500’ pattern  why?

-Normal landing discussion

-Pitch Pitch Pitch!!

-Power (power on OK)

-Stick forward (just after touchdown and slowing down…wheel landing)

-Wings level

-Go-Around!!!

-Glassy Water discussion

-Dangers of on takeoff and landing

-LVR technique

-Shoreline technique

-Instrument takeoff procedure

-Step Taxi discussion

-Speed on step (25-35 mph)

-Power required

-More going downwind

-More in turns (more power, sharper turn, sharper turn, more power)

-Outside aileron

-Confined area discussion

-60 mph over obstacle

-Drop nose to 65 mph OK

-Power in flare OK to ensure power touchdown pitch attitude

-5 SeaRey Accident Causes

-Gear down water

-GIFFTS

-Gear check base/final

-Gear up after every land takeoff.

-Weight and Balance

-C.G.

-Energy management

-Too slow=high drag=power required to maintain pitch attitude on landing

-Slow Airspeed+High Thrust Line=A LOT of up elevator on bounced landings/go-arounds to keep nose 			from diving into water/land

-Tailwheel

-Practice

-3-point after bounced wheel landing (like ANY tailwheel)

-Limitations

-Plane (1’ waves-Swell interval-crosswind land landings)

-Inexperience(25 hours on the water ASAP)

-Why is the SeaRey more difficult to fly than most aircraft

-Hi-Thrust Line (downwind descent/go-around etc.)

-Large Rudder (for water, oversize for needs in the air)

-No forward visual reference (wings level is more difficult)